| IMIVRC04 Dismantle end of life vehicles |  |
| --- | --- |

Overview

This standard is about dismantling end of life vehicles, removing vehicle parts, evaluating whether suitable for re-use, re-manufacture or re-cycling and storing in a way that will minimise damage or deterioration. It is also about ensuring accurate records are maintained throughout.

For the purposes of this standard, ELVs include light vehicles, light commercial vehicles and motorcycles.

Performance criteria

You must be able to:

P1

‍

use suitable personal and vehicle protective equipment throughout all removal activities

P2

‍

support your dismantling activities by reviewing:

P2.1

‍

vehicle documentation

P2.2

‍

vehicle technical data

P2.3

‍

removal procedures

P2.4

‍

legal requirements

P3

‍

prepare the vehicle systems and work area for safe working procedures as appropriate to the vehicle

P4

‍

prepare, check and use all the equipment required following manufacturers' instructions

P5

‍

carry out all dismantling activities following:

P5.1

‍

health, safety and environmental requirements

P5.2

‍

your workplace procedures

P6

‍

work in a way which minimises the risk of:

P6.1

‍

damage to other vehicle systems or parts

P6.2

‍

contact with leakage or hazardous substances

P6.3

‍

damage to your working environment

P6.4

‍

injury to self and others

P7

‍

identify components considered to be target parts

P8

‍

use appropriate

testing methods

to assess the performance of target

parts

to evaluate if suitable and safe for re-use or recycling

P9

‍

ensure all target

parts

conform to the vehicle operating specification and any legal requirements

P10

‍

clean and prepare

parts

for storage

P11

‍

promptly record any additional faults you notice during the course of your work to ensure unsuitable

parts

are not marked for re-use.

P12

‍

ensure all removed

parts

are tagged or labelled and stored in the correct manner

P13

‍

ensure your records are accurate, complete and passed to the relevant person(s) within the agreed timescale in the format required

Knowledge and understanding

You need to know and understand:

*Legislative and organisational requirements and procedures *

K1

‍

current relevant legislation, regulations, codes of practice and guidelines relating to end of life vehicles

K2

‍

the requirements for and importance of how to select and use personal and vehicle protective equipment when dismantling end of life vehicles

K3

‍

your organisation's systems and procedures for:

K3.1

‍

maintaining records

K3.2

‍

dealing with special considerations

K3.3

‍

the referral of problems

K3.4

‍

reporting delays to the completion of work

K4

‍

the hazards associated with working on or near petrol and alternative fuel vehicle systems and components

K5

‍

the importance of working to agreed timescales and keeping others informed of progress

K6

‍

the importance of promptly reporting anticipated delays to the relevant person(s)

Dismantling activities

*

K7

‍

the importance of checking vehicle stock records prior to starting dismantling activities

K8

‍

how to remove *

parts

without causing damage and minimising contact with leakage and hazardous substances

K9

‍

how to identify target marketable parts

K10

‍

how to evaluate whether

parts

are suitable for re-use, re-manufacture or re-cycling.

K11

‍

how to clean and prepare

parts

for storage

K12

‍

the importance of applying a unique identification number to removed

parts

K13

‍

the information to include when tagging or labelling a

part

K14

‍

the importance of ensuring

parts

are tagged with accurate information

K15

‍

the importance of ensuring the

part

tag or label is easily visible

K16

‍

how to move and store

parts

on site without causing damage or deterioration

K17

‍

the principles of

parts

grading and the circumstances where the grade might need to be amended and recorded

K18

‍

how to locate where vehicle

parts

are stored using the appropriate information systems used in your organisation

Scope/range

1.

‍

**Information** to include:

1.1.

‍

unique part identification number

1.2.

‍

originating vehicle VIN (where possible)

1.3.

‍

assigned quality grade

2.

‍

Testing methods

are:

2.1.

‍

sensory

2.2.

‍

functional

2.3.

‍

measurement

3.

‍

Parts

are:

3.1.

‍

mechanical

3.2.

‍

electrical

3.3.

‍

panels

3.4.

‍

wheels

3.5.

‍

cosmetic and trim

3.6.

‍

control systems

3.7.

‍

safety systems

Glossary

This section contains examples and explanations of some of the terms used but does not form part of the standard.

Agreed timescales

This can be as dictated by government guidance, a contractual service level agreement or customer request.

End of life vehicles

A vehicle that has come to the end of its useful life as a motor vehicle and is categorised as waste, generally due to age, malfunction or accident. These may include light vehicles, light commercial vehicles and motorcycles.

Special considerations

To include whether vehicle has been assigned to any hazard category (including flood or fire damage, biohazard contamination or an alternative fuel source), or subject to recall by manufacturer or DVSA.

Target part

A vehicle part that has demand for re-use and that the vehicle recycler removes from an ELV for re-use. May also be referred to as reclaimed, re-usable, recycled or 'green'.

Developed by

IMI

Version Number

1

Date Approved

30 Mar 2022

Indicative Review Date

30 Mar 2025

Validity

Current

Status

Original

Originating Organisation

IMI

Original URN

IMIVRC04

Relevant Occupations

Vehicle Recycler

Suite

Vehicle Recycling

Keywords

Vehicle; recycling; ELV; dismantle;