| IMIVBR05 Remove, replace and/or refit motor vehicle body panels |  |
| --- | --- |

Overview

This standard is about removing, replacing and/or refitting a combination of adjacent vehicle body panels using mechanical fastening. It also includes the application of anti-corrosion and joining materials and ensuring correct alignment with adjacent panels and fittings.

Performance criteria

You must be able to:

P1

‍

identify component

materials

involved in the construction of the vehicle in the areas that will be worked on during repair, prior to working on the vehicle

P2

‍

use the appropriate personal protective equipment (PPE) when removing and replacing

body panels

*

P3

‍

protect the vehicle, its contents and systems effectively when removing and replacing *

body panels

*

P4

‍

inspect, prepare and use all the *

tools and equipment

required following manufacturers' instructions

P5

‍

remove, replace and/or refit all necessary

body panels

and components following:

P5.1

‍

the manufacturer's

methods

/instructions

P5.2

‍

recognised researched repair

methods

P5.3

‍

your workplace procedures

P5.4

‍

the vehicle work specification

P5.5

‍

health, safety and legal requirements

P6

‍

promptly seek assistance from the relevant person(s)  where there is the potential for your work to disturb other vehicle systems

P7

‍

store all removed components safely in the correct location and in accordance with relevant legislation

P8

‍

use replacement

body panels

and components which conform to the vehicle specifications for dimensions,

materials

and functional capability

P9

‍

use and apply body component anti-corrosion

materials

and joining

materials

conforming to the manufacturer's specification

P10

‍

ensure panels are removed and replaced minimising damage to mating surfaces (any damage caused should be correctly reinstated)

P11

‍

ensure panels are replaced without incurring damage to the vehicle systems

P12

‍

ensure all refitted

body panels

and components are aligned correctly with adjacent panels and fittings

P13

‍

complete all activities within the agreed timescale

P14

‍

promptly report any anticipated delays in completion to the relevant person(s)

Knowledge and understanding

You need to know and understand:

K1

‍

the health and safety legislation and workplace procedures relevant to workshop practices and personal protective equipment (PPE)

K2

‍

the agreed vehicle work specification

K3

‍

the importance of working to agreed timescales and keeping others informed of progress

K4

‍

the relationship between time, cost and profitability

K5

‍

the requirements for protecting the vehicle and contents from damage before, during and after removing and

re

*

fit

ting

activities

K6

‍

your workplace procedures for the referral of problems and reporting delays to the completion of work

K7

‍

the importance of promptly reporting anticipated delays to the relevant person(s)

K8

‍

how to prepare, test and use the

tools and equipment

required for the removal, replacement and/or

refitting

of

body panels

and ancillary fittings

K9

‍

the properties of component

materials

involved in the construction of the vehicle in the areas that will be worked on during repair

K10

‍

the properties and safe use of body component sealants, adhesives and anti-corrosion

materials *

K11

‍

the type of sealants and anti-corrosion

*materials

to use and the manufacturer's recommended methods for their application and thickness

K12

‍

how to apply sealants and anti-corrosion

materials *

K13

‍

how to interpret and use sources of information relevant to the removal and **refitting

of

body panels

K14

‍

the need for correct alignment of panels and the

methods

used to achieve this

K15

‍

the types of quality control checks that can be used to ensure correct alignment and contour of panels and operation of components to manufacturer's specification

K16

‍

how to work safely avoiding damage to the vehicle and its systems

K17

‍

the methods of storing removed components and the importance of storing them correctly and in accordance with legal requirements

K18

‍

the removal and replacement procedures for

body panels

using mechanical fastening techniques

K19

‍

how panel removal and

refitting

affects the overall body structure of the vehicle

K20

‍

the manufacturer's approved

methods

of working for the removal and replacement of

body panels

K21

‍

the difference between the types of replacement parts available

K22

‍

the potential implications of fitting a recycled or refurbished part

Scope/range

All of the items listed below form part of this National Occupational Standard.

1.

‍

Body panels

include combinations of 3 or more adjacent panels (Examples of this include: two doors and a wing; two wings and a bonnet; bonnet, wing and door on the same side; bumper, wing and bonnet; bonnet, wing and bolted on front panel).

2.

‍

Materials

include all component materials in the repair area.

3.

‍

Fitting methods

include mechanical fastening.

4.

‍

Tools and equipment

are: