| IMPMC122 Carry out flour packing, storage and despatch in milling |  |
| --- | --- |

Overview

This standard is about the skills and knowledge needed for you to carry out flour packing, storage and despatch in milling. The correct production of packs and understanding the importance of this is essential to maintaining the integrity of the product. It is therefore essential to maintaining product quality and safety. You will need to understand the purpose of coding and labelling information and what the correct actions to take are when there is an interruption to the supply of products. You will need to understand the principles of packing flour, storage and despatch of flour from mills. You will also need to understand the different types of packing operations, palletising, warehouse stacking/trucking and despatch control. This standard is for you if you work in food and drink manufacture and/or supply operations and you are involved in carrying out packing operations.

Performance criteria

You must be able to:

Prepare to produce individual packs

- prepare to produce individual packs following health, safety, food

- safety and hygiene standards

- check packing specifications within agreed timescales

- set up equipment according to specification

- check that there is sufficient packing material available and that it

- is fit for use

- check that services meet requirements

- check that the product to be packed is available and fit for use

- take action in response to operating problems

- maintain communication with the relevant people

- Carry out production of individual packs

- carry out production of individual packs following health, safety,

- food safety and hygiene standards

- use equipment in accordance with health and safety procedures

- and check that it is supplied with materials

- monitor pack quality and quantity and take action in response to

- defects

- achieve the required quality and quantity of output within the

- specified time

- check that there is minimal waste

- maintain communication with the relevant people

- Finish production of individual packs

- finish production of individual packs following health, safety, food

- safety and hygiene standards

- dispose of surplus consumables in compliance with company

- procedures

- deal with waste, scrap and non-standard products in compliance

- with company non-conforming material/product procedures

- stop the packaging run at agreed time

- make equipment ready for future use after the completion of the

- process

- maintain communication with the relevant people

- complete all documentation and records within agreed timescales

- and in compliance with company procedures

- Perform checks before carrying out palletising and wrapping

- follow the required health and safety, food safety, hygiene,

- security and environmental standards and instructions

- receive and confirm palletising and wrapping instructions

- check that palletising and wrapping materials and products are

- available

- check that equipment and services are available and fit for use

- identify the product to be palletised and wrapped

- check that equipment is in the correct operating condition and

- ready for use

- Carry out palletising and wrapping

- start equipment in the correct order and use it in accordance with

- company procedures

- check that products remain within specification and take

- appropriate action to correct those which do not

- Finish palletising and wrapping

- follow the required health and safety, food safety, hygiene,

- security and environmental standards and instructions

- achieve the required output to the correct specification

- control palletising and wrapping materials to match the packing

- run and return surplus to the designated location

- shut down equipment in accordance with company procedures

- deal with waste or scrap in accordance with company procedures

- check the equipment is made ready for the next production run

- make product ready for movement to the designated location

- complete all documents and records in accordance with company

- procedures and within agreed timescales

- maintain communication as required

- Check that the process works efficiently

- monitor and replenish packing materials as required

- maintain required output rates

- check that services, materials and equipment are used

- economically

- complete all documents and records within agreed timescales and

- in accordance with company procedures

- maintain communication as required

Knowledge and understanding

You need to know and understand:

- what the typical stages are in the packing process for bagged

- products

  - what the functions of magnets and redressers are in packing

  - the types of feed arrangement for weighers, and how the

- mechanical and electronic load cell weigher systems function

  - the importance and function of the in-line check weigher

  - what the capabilities and advantages of the three types of bag or

- sack packers are

  - the functions of bag placers, possing, bag closure and metal

- detectors

  - how bags or sacks need to be marked and comply with labelling

- regulations

  - what the important cost and effectiveness features of sack design

- and function are

  - the advantages and features of filling and using minibulk

- containers (FIBC)

  - how manual/automated palletisation works, their advantages and

- disadvantages

  - features of warehouse design and layout, including multistorey/single storey

  - what the three types of unit load storage are and their principle

- features

  - the comparative features and functions of forklift trucks used in

- milling storage

  - the applications for hand operated/pedestrian and turret trucks

  - what the training and safety requirements are for operating lift

- trucks

  - what needs to be taken into account when planning a warehouse

- system

  - how stock control, bulk/bag stocks goods flow and orders are

- managed

  - what the vehicle weight restrictions and limits are for loading mill

- products

  - the features and principles of loading bagged and bulk flour

  - how deliveries of flour to customers are organised and controlled

  - the features of unloading bulk and bagged flour

Developed by

Improve

Version Number

2

Date Approved

01 Jan 2019

Indicative Review Date

01 Jan 2024

Validity

Current

Status

Original

Originating Organisation

Improve

Original URN

impsd309 (skills), impsd316 (skills), impmc122k

Relevant Occupations

Elementary Goods Storage Occupations, Elementary Process Plant Occupations, Food Preparation Trades, Manufacturing Technologies

Suite

Milling and Cereals

Keywords

Food; Drink; Manufacturing; Milling; Flour; Packing; Storage; Despatch; Weigher; Bulk; Bagged; Trucks