| PROUSF01 Produce lined curtains with heading tape |  |
| --- | --- |

Overview

The standard covers the skills and knowledge required to complete lined curtains with heading tape, mitres, hand stitching and pattern matching

This includes:

- producing precise mitred corners

- positioning and securing weights

- positioning and securing lining

- applying hand-stitching techniques

- producing pattern match samples

The standard is aimed at those working in soft furnishings.

Performance criteria

You must be able to:

- comply with health and safety requirements and procedures at all times

- obtain specifications and requirements of the items to be produced following standard operating procedures

- carry out your work to the required quality and output to meet production schedules and targets

- select fabrics for curtain construction to meet specifications

- select curtain linings to meet specifications

- evaluate materials for their suitability for use in the construction of pattern match curtains

- evaluate patterns on fabric in order to provide the appropriate positioning on a finished curtain

- sew multiple widths of fabric in the same direction to meet specifications

- construct curtains using exact measurements and methods

- apply stitched, tension and method of machine finishes to the sections of curtains following standard operating procedures

- construct true mitred corners using precise measurements to meet specifications

- cover weights following standard operating procedures

- position and secure weights to curtain corners and seams following standard operating procedures

- position and secure lining with interlocking stitched positioned following standard operating procedures

- construct hems without visible disturbance to the front of curtains following standard operating procedures

- check there is no visible disturbance to the front of fabrics

- apply slipstitching following standard operating procedures

- apply ladder stitching to mitred corners following standard operating procedures

- apply herringbone stitching following standard operating procedures

- finish all stitching to meet specifications

- pattern match fabrics following standard operating procedures

- secure heading tapes to main fabrics to meet specifications

- secure draw strings under the end of the tape leaving the opposite end free to gather up the curtain

- stitch through layers of fabric without causing disturbance to quality and finish of fabrics

- press fabric seams using the appropriate iron temperature for fabrics

- carry out your work to the required quality and output to meet production schedules and targets

- identify any problems and deal with them according to standard operating procedures

- record information in the appropriate information systems

Knowledge and understanding

You need to know and understand:

- relevant health, safety and environmental requirements, statutory regulations and industry standards/codes of practice that have to be complied with

- how to confirm specifications for work being undertaken

- level of detail required in specifications

- the use of true and false mitred corners

- how to construct true mitred corners using precise measurements

- commercially covered weights

- appropriate stitches, tension and methods to hand finish samples

- materials used for the construction of pattern matches

- materials and their suitability for use in the construction of pattern match samples

- how to evaluate patterns on fabric in order to provide the appropriate positioning on finished curtains

- how to press fabric seams

- type of problems that can occur and the standard operating procedures for dealing with them

- information systems used and why it is important to record information

- selection of suitable fabrics for curtain construction

- selection suitable curtain linings

- construction of curtains using exact measurements and methods

- stitched, tension and methods of machine finishing

- positioning and securing of lining with interlocking stitch

- methods to construct hems without visible disturbances

- construction of slipstitching, ladder stitching and herringbone stitching

- securing heading tapes to fabric and types of heading tapes available

- method of securing draw strings

Developed by

Improve

Version Number

3

Date Approved

31 Jan 2020

Indicative Review Date

31 Jan 2025

Validity

Current

Status

Original

Originating Organisation

Proskills

Original URN

PROUSF01

Relevant Occupations

Furniture Makers and Other Craft Woodworkers, Upholsterers

Suite

Upholstery & Soft Furnishings

Keywords

lined curtains, heading tape, fabric, upholstery, soft furnishings,