| IMPPO248 Receive and offload bulk liquids in food and drink operations |  |
| --- | --- |

Overview

​

This standard is about the skills and knowledge needed for you to

receive and offload bulk liquids in food and drink manufacture and/or

supply operations.

Receiving and offloading bulk liquids including raw milk, cream,

concentrated milk, syrup and oil is important to the maintenance of

production schedules and the maximisation of yield and quality from

incoming materials. You will need to be able to ensure the tanker bay

and hosing equipment is available to receive the tanker. You will also be

able to check that the offloading equipment is ready and in working

order. You must also be able to adhere to organisational and regulatory

requirements and address problems within the limits of your

responsibility. Complying with and understanding health and safety, food

safety, environmental and organisational requirements are essential features of this

standard.

This standard is for you if you work in food and drink manufacture and/or

supply operations and are involved in receiving and offloading bulk

liquids in food manufacture.

Performance criteria

You must be able to:

1. check the liquid matches delivery schedule documentation

2.  check product temperature and record details according to organisational

requirements

3. take pre-delivery samples for testing adhering to organisational hygiene and

sampling procedures

4. take action to address problems

5. check that there is enough storage space available for the expected delivery

volume

6. check offloading pumps and hoses are available and clean

7. check that the tank or silo is clean and fit for use

8. check that the correct routes have been set to pump the product to the

correct tank or silo

9. check that all drain valves and inspection hatches are securely closed

10. inform relevant people when unloading is about to begin

11. check for leaks during offloading

12. check that off-loading is complete

13. inform relevant people when off-loading is complete

14. work within the limits of your responsibility and take action to address

problems

15. store pumps and hoses according to organisational requirements

16. complete del

ivery documentation to organisational requirements

Knowledge and understanding

You need to know and understand:

​

1. the organisational procedures relating to the receipt of bulk products

2. how to access delivery schedules documentation that tells you the quantity

and type of deliveries to expect

3. the communication structures and procedures within the organisation and

why it is important to follow them

4. how to check equipment is clean and suitable for use and what may happen

if they are not carried out

5. the organisational requirements for testing on pre-delivery liquids

6. acceptable reasons for refusing bulk products into storage

7. organisational procedures to follow if faults are found in the liquid

8. what the correct routing is to transfer bulk product from tanker to storage

vessel

9. how to deal with waste or spillage and what may happen if this is not done

10. what to do if there is contamination or loss of bulk product

11. how to check for leaks during the offloading process

12. why it is important to store hoses and other equipment

13. how to work within the limits of your responsibility and take action to

address problems

14. organisational requirements for the completion of documentation and why it

is important to follow them

15. how factory digitisation supports the production process

Developed by

Improve

Version Number

4

Date Approved

30 Mar 2023

Indicative Review Date

30 Mar 2028

Validity

Current

Status

Original

Originating Organisation

Improve

Original URN

IMPPO248

Relevant Occupations

Engineering and Manufacturing Technologies, Manufacturing Technologies, Plant and Machine Operatives, Process Operatives, Process, Plant and Machine Operatives

Suite

Production and Control Operations in Food Manufacture

Keywords

Food, manufacture, dairy, liquids, bulk, offload