| SFHOH12 Take an oral tracing registration |  |
| --- | --- |

Overview

This standard covers the safe practice of taking oral tracing records to produce transfer records as required for mounting casts on articulators. This function will be performed to check on occlusal problems prior to making decisions about an appropriate treatment plan. Users of this standard will need to ensure that practice reflects up to date information and policies.

Performance criteria

You must be able to:

"1. access and accurately interpret all relevant work instructions and information

2. work safely at all times and in accordance with all relevant legislation, guidelines, policies, procedures and protocols

3. deal promptly and effectively with any problems within your control and report those which cannot be solved

4. identify and minimise hazards and risk in the workplace

5. confirm identity of the individual in accordance with organisational procedures

6. gain valid, informed consent from the individual in accordance with specific procedures

7. communicate with relevant others at a pace, manner and level appropriate to their understanding, preferences and needs

8. respect the individual's rights and wishes relating to their privacy, beliefs, and dignity

9. provide support to the individual and ensure health and safety measures are implemented at all times

10. identify pre-disposing clinical conditions that could affect the use of a face bow on the individual's face, taking account of individual's safety and/or accuracy requirements

11. decide on a suitable and safe technique related to the type of face bow to be used, and whether the individual is edentulous, partially edentulous or dentate

12. explain the procedure clearly to the individual

13. apply standard precautions for infection prevention and control

14. prepare the equipment needed, and any required materials, in accordance with the manufacturer's instructions

15. position the individual comfortably for the procedure and ensure their auditory canals and facial tissues are protected

16. identify relevant anatomical features and place the face bow and locate it appropriately on the individual's face

17. adjust both the occlusal registration fork and orbital reference plane indicator on the face bow and lock in place to obtain an accurate record, taking due care of the individual's tissues

18. remove the face bow from the individual and check the record for accuracy and coverage

19. discuss the record with the individual, and explain the next stage in the process

20. decontaminate the face bow

21. link the face bow record with any associated intra-oral records, if appropriate

22. take adequate precautions to store the registration in an appropriate container that will preserve the record until it is used

23. dispose of waste in accordance with organisational procedures

24. complete and store all relevant documentation in accordance with organisational requirements"

Knowledge and understanding

You need to know and understand:

"1. the current legislation, guidelines, policies, procedures and protocols which are relevant to your work practice and to which you must adhere

2. the scope and limitations of your own competence, responsibilities and accountability as it applies to your job role

3. how to access and interpret all relevant work instructions and information

4. specific procedures for reporting issues which are beyond your competence, responsibilities and accountability

5. the duty to report any acts or omissions that could be unsafe/detrimental to you or others

6. the hazards and risks which may arise during the execution of your work role and how you can minimise these

7. how to communicate with relevant others at a pace, manner and level appropriate to their understanding, preferences and needs

8. the correct use of any equipment and PPE to protect the health and safety of you and others

9. how to obtain positive confirmation of an individual's identity in accordance with organisational procedures

10. the principles, practice and procedures associated with informed consent

11. the importance of applying standard infection control precautions to taking oral recordings, **and the potential consequences of poor practice

12. relevant anatomy and physiology related to the temporomandibular joint (TMJ) articulations and occlusions and related disorders

13. procedures for taking oral tracing records

14. the range of oral tracing systems currently available

15. clinical practice and procedures related to taking oral tracings on a dentate, partially dentate and edentulous individual, including how to use articulators

16. how to manage and integrate the procedure of oral tracing in providing functional and aesthetic dental prostheses (fixed and removable) with individual requirements and/or needs

17. how to measure associated records with hybrid oral tracing devices, or similar

18. the developments of information communication technology (ICT) in respect of recording aspects of TMJ location

19. how to dispose of waste in accordance with organisational procedures

20. how to complete and safely store all relevant documentation in accordance with organisational requirements"

Developed by

Skills for Health

Version Number

3

Date Approved

30 Mar 2022

Indicative Review Date

30 Mar 2027

Validity

Current

Status

Original

Original URN

SFHOH12

Relevant Occupations

Dental Nurse, Dental Assistant, Dental Technician

Suite

Dental Technology

Keywords

Oral, tracing, registration