| SEMPW309 Assess and prepare permanent way materials, components and equipment for moving |  |
| --- | --- |

Overview

This standard is about assessing and preparing permanent way materials, components and equipment for moving to ensure that the actual movement will be done safely and correctly.

You will be required to select the appropriate moving methods and techniques to lift and move permanent way assets safely in accordance with the lifting plan. These methods and techniques may be manual or mechanical and consideration must be given to the nature of the load and its intended destination. In order to lift, move and handle equipment you must understand the level and extent of your authority and responsibility.

The quality standards and accuracy to be achieved will be in line with your organisation's procedures. You will be expected to work within approved procedures and specifications and will work within the limits of your own responsibility.

Your underpinning knowledge will provide a good understanding of your work and will provide an informed approach to applying problem-solving techniques and procedures when assessing and preparing permanent way materials, components and equipment for moving.

You will comply with any organisational procedures, and legislative or regulatory health, safety and environmental requirements applicable when assessing and preparing permanent way materials, components and equipment for moving.

This standard is for all skilled operatives engaged in the engineering activities associated with the movement of permanent way materials, components and equipment.

Performance criteria

You must be able to:

P1 maintain safe working practices and comply with all relevant health and safety regulations, directives, and guidelines

P2 identify any necessary changes to safety requirements upon site arrival and report to the relevant person(s) without delay

P3 source and follow the appropriate schedules, diagrams and related specifications for the work being carried out as approved by your organisation

P4 establish the weight of the load to be moved

P5 establish the characteristics of the load to be moved

P6 follow the lifting plan and identify suitable equipment to move the load

P7 assess the equipment to be used to confirm it is capable of moving the load

P8 ensure the planned route for moving the load is still suitable, ensuring minimum risk to people and property

P9 ensure that the load is correctly loaded, secured and protected before moving, in accordance with your organisations procedures

P10 prepare appropriate documentation to update on progress and report completion of activities in line with your organisation's procedures

P11 address problems within your control and report unresolved issues

Knowledge and understanding

You need to know and understand:

K1 the relevant health and safety regulations, directives, guidelines, and safe working practices and procedures defined by your organisation, as appropriate to the activity and your working area

K2 your responsibilities in respect of health, safety and environment, including the limits of your personal responsibility, your legal responsibility for your own health and safety and the health and safety of others

K3 the importance of wearing protective clothing and other appropriate type of safety equipment (PPE) and where to obtain it from

K4 the hazards associated with working on permanent way infrastructure

K5 how to locate and access the site

K6 how to source and follow schedules, diagrams and specifications relevant to your role, as approved by your organisation

K7 how to locate and identify the component(s) to be lifted/moved

K8 what information is required prior to moving materials, components and equipment

K9 the type of lifting, moving and handling equipment to be used including:

K9.1 rail/timber nips

K9.2 jacks

K9.3 grabs

K9.4 slings

K9.5 iron men

K9.6 trolleys

K9.7 rail scooters/skates

K9.8 cranes

K9.9 road/rail vehicles

K9.10 hand tools

K9.11 frame with lifting equipment

K10 the range of methods and techniques for lifting, slinging, moving and handling materials, components and/or equipment approved by your organisation including:

K10.1 hand signals

K10.2 dealing with obstructions

K10.3 securing of loads

K11 how to identify the different characteristics of the loads to be moved, including:

K11.1 weight distribution

K11.2 shape/size/length

K11.3 vulnerability to damage

K11.4 alternative methods of lifting

K12 how to determine the weight of loads

K13 the methods and techniques for securing and protecting a load

K14 the methods and techniques for route planning should cover:

K14.1 safety considerations

K14.2 obstructions

K14.3 fouling other assets and components

K14.4 the most direct route

K15 your organisation's procedures for the use, care and control of lifting equipment

K16 the relevant data management processes and technologies for recording information

K17 the relevant reporting lines, timeframes and procedures as approved by your organisation

K18 the impact of your work on the operations of other departments and the impact of their work on you

K19 the limits of your own authority and responsibility and those of others involved

Developed by

NSAR

Version Number

4

Date Approved

30 Apr 2024

Indicative Review Date

01 May 2027

Validity

Current

Status

Original

Originating Organisation

SEMTA

Original URN

SEMPW309

Relevant Occupations

Rail Engineering

Suite

Rail Engineering

Keywords

Moving; permanent way; equipment; assess