| SFHOH16 Design and manufacture removable polymeric prostheses |  |
| --- | --- |

Overview

This standard focuses on the design and manufacture of partial and complete removable prostheses - polymeric dentures using manual or digital techniques. These are dental devices which are custom-made to fit the individual's unique mouth shape and which replace either a partial or complete set of upper and/or lower teeth. The prosthesis may incorporate a range of factors including, but not limited to:

* a method of retention

* aesthetics including the creation of an overdenture

* the degree of occlusion including the use of onlays, deep overbite

* the requirement for minor obturation

The removable prostheses may be new, replacement, immediate or duplicate and may contain resilient liners. You need to design and manufacture a trial prosthesis, modify these where necessary following a try-in and then manufacture, and finish final removable prosthesis.

The term 'client' is used to mean the member of the oral health care team who has prescribed the custom-made prosthesis. Clients may be external to the organisation (such as other laboratories, dental practitioners, training schools) or internal (within a dental hospital). The individual is the one for whom the custom-made prosthesis is being made.

The design and manufacturing process of dental prosthesis may be carried out in a regulated dental laboratory within a variety of settings.

Users of this standard will need to ensure that practice reflects up to date information, policies and regulations.

Performance criteria

You must be able to:

- communicate with relevant others at a pace, manner and level appropriate to their understanding, preferences and needs

- collate and confirm accuracy of all specification information required for prostheses design and manufacture with relevant others

- interpret and analyse information captured of both soft and hard tissues in oral environment using both analogue and digital techniques

- select components, materials, and equipment, ensuring their suitability .

- set up and operate the manufacturing equipment in accordance with the specification

- manufacture removable prostheses using suitable manual or digital techniques adjusting manufacturing processes as necessary.

- devest or retrieve the manufactured product using an appropriate method which releases the item without causing damage

- ensure that the manufactured product matches the specification and make any necessary adjustments

- clean and finish the restoration, prepare and package it safely for dispatch together with instructions for the individual and client

- dispose of waste in accordance with all relevant legislation, guidelines, and workplace procedures

- complete and store all documentation in accordance with relevant legislation, guidelines, and workplace procedures

Knowledge and understanding

You need to know and understand:

- how to communicate with relevant others at a pace, manner and level appropriate to their understanding, preferences and needs

- the importance of applying standard infection control precautions and the potential consequences of poor practice

- the role of removable prostheses in the restoration and maintenance of:

- • tissue support

- • aesthetics

- • phonetics

- • function of occlusion and the temporomandibular joint

- the principles, uses, methods, techniques and equipment involved in digital design and manufacturing.

- how to select the necessary components, materials and equipment and confirm that they are fit for purpose

- skeletal anatomy, tooth morphology, orofacial musculature and temporomandibular joint function and movement

- the classification, aetiology, including oral cavity disorders and diseases and the physiological effects of malocclusions.

- the physiological and pathological changes associated with ageing process and trauma related to the oral environment

- the importance of retention of the periodontal ligament and the changes in proprioception due to loss of periodontal ligament

- the emotional response by the individual to tooth loss

- the importance of restoring and maintaining the occlusal vertical dimension

- the benefits and restrictions of immediate tooth replacement in the provision of removable prostheses

- the benefits and restrictions of retaining root structures in the provision of removable prostheses

- the use and need for transitional removable prostheses

- the modern concepts for the use of resilient liners and tissue conditioners

- the design limitations of large anterior undercuts and pre-existing dental conditions

- the use of digital representations or a dental surveyor and identifying useable and non useable undercuts in the design of the dentures

- the principles and practice of:

- • retention and stability

- • aesthetics and phonetics

- • articulation

- the principles of removable prosthesis design

- the classification and sub-classification of materials on the basis of chemical composition and internal structure

- the mechanical, physical, thermal, chemical and biological properties of materials

- the purpose of different products used for cast and mould manufacture or digital representation

- the purpose of different materials used in the manufacture of removable prostheses

- legal and physical implications of modifying manufacturer products and ensuring quality assurance .

- how to clean and finish the restoration, prepare and package it safely for dispatch together with instructions for the individual and client

- different methods of waste disposal and how to apply these

- the importance of updating documentation and storing individuals records safely and securely

Developed by

Skills for Health

Version Number

4

Date Approved

20 Mar 2025

Indicative Review Date

20 Mar 2030

Validity

Current

Status

Original

Originating Organisation

Skills for Health

Original URN

SFHOH16 ; SFHOH17

Relevant Occupations

Associate Professionals and Technical Occupations, Dental Technician

Suite

Dental Technology

Keywords

Complete, removable, prosthesis; Digital